Knowledge Bites

Current Rebar Sales Codes (Regular Production)

| Diameter | Length | Material Code | Family Code | Grade |
| --- | --- | --- | --- | --- |
| millimetre | metre |  |  |  |
| 10 | 12 | WR541001200 | B100C | B500C |
| 12 | 6 | WR541200600 | B120C |  |
|  | 12 | WR541201200 |  |  |
|  | 14 | WR541201400 |  |  |
| 16 | 6 | WR541600600 | B160C |  |
|  | 12 | WR541601200 |  |  |
|  | 14 | WR541601400 |  |  |
|  | 15 | WR541601500 |  |  |
| 20 | 6 | WR542000600 | B200C |  |
|  | 12 | WR542001200 |  |  |
|  | 14 | WR542001400 |  |  |
|  | 15 | WR542001500 |  |  |
|  | 16 | WR542001600 |  |  |
| 25 | 12 | WR542501200 | B250C |  |
|  | 14 | WR542501400 |  |  |
|  | 15 | WR542501500 |  |  |
|  | 16 | WR542501600 |  |  |
|  | 18 | WR542501800 |  |  |
| 32 | 12 | WR543201200 | B320C |  |
|  | 14 | WR543201400 |  |  |
|  | 15 | WR543201500 |  |  |
|  | 16 | WR543201600 |  |  |
|  | 17 | WR543201700 |  |  |
|  | 18 | WR543201800 |  |  |
| 40 | 12 | WR544001200 | B400C |  |
|  | 14 | WR544001400 |  |  |
|  | 15 | WR544001500 |  |  |
|  | 16 | WR544001600 |  |  |
|  | 17 | WR544001700 |  |  |
|  | 18 | WR544001800 |  |  |

Those are the standard production lengths acknowledged by the mill.

The rolling of what is termed long or extender lengths, (the term relates to transport requirements), is planned according to orders placed by the independent market sector and a forecast of requirements by CSS (Group) through to next planned, (anticipated), rolling dates of said diameter.

The mill is practically capable of rolling any lengths to a single decimal place between 6metre & 18metre. For example 10.0metre, 13.5metre, 17.5metre etc...

These ‘non-standard lengths’ are rolled strictly to order.

The mill is also able to roll 50mm diameter strictly to order.

| Diameter | Length | Material Code | Family Code | Grade |
| --- | --- | --- | --- | --- |
| millimetre | metre |  |  |  |
| 50 | 12 | WR545001200 | B500C | B500C |
|  | 14 | WR545001400 |  |  |
|  | 15 | WR545001500 |  |  |
|  | 16 | WR545001600 |  |  |
|  | 17 | WR545001700 |  |  |
|  | 18 | WR545001800 |  |  |

This one diameter can require a significant number of non-standard or make to order lengths within the rolling sequence.

The Reason?

Well, a 1.5-metre offcut from a 16mm diameter bar weighs 2.37 kilograms. The same offcut of 50mm diameter weighs 23.1 kilograms, a significant loss in monetary terms.