Technical and Commercial Report

Virgin PET (vPET) and Recycled PET (rPET) are Distinct

Evidence relevant to product scope, supply, demand, injury and remedy design

Contents

EXECUTIVE SUMMARY	3

Method and evidentiary hierarchy	3

A.	INDUSTRIAL SYSTEM AND PRODUCTION ECONOMICS	3

Simplified Industrial Pathways	3

Shared polymer does not imply a shared production market	4

Illustrative cost sensitivity	5

B.	TECHNICAL CHARACTERISTICS AND QUALIFICATION	6

Equivalent end-use performance must be demonstrated, not assumed	6

Balanced technical conclusion	7

C.	COMMERCIAL IDENTITY, DEMAND AND SUBSTITUTION	7

rPET is bought for a verified attribute that vPET cannot provide	7

Policy is changing the demand curve	8

Commercial test for interchangeability	8

D.	PRICING ARCHITECTURE AND PRICE TRANSMISSION	9

Separate price series exist because the costs and markets differ	9

A concrete 2025 price snapshot	9

E.	UK SUPPLY, DEMAND, CAPACITY AND MARKET EVOLUTION	9

The UK has separate vPET and rPET capacity constraints	9

F.	CONSEQUENCES FOR IMPORT, INJURY AND REMEDY ANALYSIS	10

A combined PET dataset can generate the wrong causal story	10

The legal/economic proposition	11

Likely price effect of a broad safeguard	11

G.	RECOMMENDED TRA ANALYTICAL FRAMEWORK	11

Minimum segmentation needed for a reliable determination	11

Evidence TRA should request and publish in non-confidential form	11

Customs-administerable evidence for rPET	12

OVERALL CONCLUSION	12

References	13

# EXECUTIVE SUMMARY

vPET and rPET may share the same underlying polymer chemistry and overlap in end use, but they are not a single homogeneous commercial product. They originate from fundamentally different feedstock systems, require different production assets and process controls, have different cost structures and pricing dynamics, face different quality and regulatory risks, and are purchased for different commercial and policy reasons. Aggregating them in a safeguard assessment risks conflating policy-driven demand for rPET with displacement of UK vPET production, and treating injury to one domestic industry as though it were injury to another.

This distinction remains important even where vPET and rPET ultimately perform the same function, such as in beverage bottle packaging. Similar distinctions exist across other materials industries. Primary and secondary aluminium, for example, may serve many of the same downstream applications, but primary aluminium is produced from alumina through electrolytic smelting, while secondary aluminium is produced from recovered scrap through remelting and refining. Comparable distinctions exist between virgin and recycled paper, primary and scrap-based steel, and virgin and recycled glass and polymers. In each case, similar downstream functionality does not eliminate fundamental differences in feedstocks, manufacturing processes, cost structures and supply chains.

rPET is analogous. Its economic and industrial identity is rooted in a recycling value chain built around the collection, sorting and processing of recovered PET, rather than the PTA/MEG polymerisation value chain used to manufacture vPET.

For clarity, unless otherwise stated, “rPET” in this report refers to mechanically recycled, post-consumer, bottle-grade PET. Chemically recycled PET follows a distinct production route and supply chain and, at present, remains relatively limited in scale and generally more costly than mechanically recycled rPET.

## Method and evidentiary hierarchy

The analysis triangulates: (i) official UK trade-remedy and tax/policy sources [4] [5] [6] [7]; (ii) peer-reviewed polymer/recycling research; (iii) independent commodity-price assessments; and (iv) company capacity disclosures [3] [2] [1].

# INDUSTRIAL SYSTEM AND PRODUCTION ECONOMICS

## Simplified Industrial Pathways

Figure 1: Virgin PET (vPET) Production Value Chain – From Petrochemical Feedstocks to Virgin Resin

Figure 2a: PET Collection Value Chain – From Waste to Post-Consumer Bales

Figure 2b: Recycled PET (rPET) Production Value Chain – From Post-Consumer PET Bales  to Recycled Resin

## Shared polymer does not imply a shared production market

Virgin PET is created from Purified Terephthalic Acid (PTA) and Monoethylene Glycol (MEG) through polymerisation and finishing [8]. Mechanical rPET starts with used PET articles and adds an upstream recovery chain before extrusion, filtration, pelletising and, for bottle-grade material, decontamination and solid-state polycondensation (SSP) [9] [10] [11]. Primary economic dimensions are further tabulated below.

| Economic dimension | vPET | rPET |
| --- | --- | --- |
| Feedstock market | PTA and MEG; globally traded petrochemical inputs. | Post-consumer/post-industrial PET, bales or flakes; local collection and sorting quality matter. Seasonal collection patterns, bottle separation and segregation processes impact feedstock availability. |
| Asset base | Esterification/polycondensation, melt phase, pelletisation, crystallisation, solid-state polymerisation (SSP). | Collection/sorting interfaces, wash line, flake preparation, decontamination, extrusion, melt filtration, pelletisation and SSP. Recycling capacity depends on the smallest capacities across sorting, washing, decontamination, and polymer processing |
| Material yield | Feedstock coefficients are relatively stable about 0.84 t PTA and 0.34 t MEG per tonne of PET. PTA is about 0.66 t of PX (Paraxylene) | Yield depends on caps, labels, dirt, non-PET polymers and process losses. Yield could also vary from bale to bale. PET bale quality can be highly variable. Reprocessing yields can fall to around 50% for lower-quality mixed-collection bales, while even well-sorted material typically experiences meaningful losses from labels, caps, non-PET plastics, trays, dirt, moisture and other contaminants. High-quality bottle bales may achieve reprocessing yields in the 80–85% range, making bale quality and yield a major driver of rPET production cost. |
| Main variable costs | PTA, MEG, energy, catalyst/additives, conversion, freight. | Bale/flake price, collection/sorting, chemicals, water, energy, yield loss, filtration, decontamination/SSP, testing and certification. Food-grade rPET requires more rigorous decontamination and qualification than fiber-grade material, adding processing cost and quality risk. Lower-priced rPET may reflect less stringent processing or specifications, though price alone is not a reliable indicator of quality. |
| Supply geography | Integrated regional/global petrochemical network. Well established, routinely traded and transported global petrochemicals | Collection is inherently tied to waste systems; food-grade output depends on feedstock provenance and approved recycling capacity. Bales or flakes can be transported long distances, but economics deteriorate quickly, generally favoring feedstock sourced close to rPET conversion facilities. |
| Scale response | High fixed-cost continuous plant; output can rise within available line capacity. | New output requires both plant capacity and sufficient clean, qualifying bale feedstock; the two do not expand automatically together. |
| Core economic risk | Feedstock and energy volatility; utilisation and plant reliability. | Feedstock scarcity/quality, contamination, mass yield, qualification, collection design and policy demand. Variable rPET quality can create significant operating risk, including metal contamination, inconsistent viscosity and drying, higher scrap and defects, process adjustments, reduced throughput, and lost machine time. |

## Illustrative cost sensitivity

For vPET, a £100/t change in PTA changes the feedstock component by about £84/t; the same change in MEG changes it by about £34/t, before conversion and logistics. For rPET, a £100/t change in bale/flake feedstock translates to roughly £167/t of finished output cost at 60% yield, before washing, decontamination, extrusion and SSP. These pricing mechanics are fundamentally different.\

# TECHNICAL CHARACTERISTICS AND QUALIFICATION

## Equivalent end-use performance must be demonstrated, not assumed

High-quality food-grade rPET can be safely used at high recycled-content levels, but it reaches that outcome through process-specific feedstock controls, challenge testing, decontamination and qualification. That is commercially different from purchasing a conventional vPET grade. EFSA identifies post-consumer input quality, drying/crystallization, extrusion/crystallization and SSP operating conditions as critical for decontamination performance for two commercially established mechanical recycling technologies that use broadly the same pathway but different process design and decontamination configurations. [12] [13]

Further technical differences are outlined below, together with their commercial implications.

| Specification / control | Typical vPET position | rPET-specific variability or control | Commercial consequence |
| --- | --- | --- | --- |
| Intrinsic viscosity (IV) | Built and controlled through polymerisation/ SSP. | Prior thermal history and degradation must be restored/managed; contaminants can retard chain rebuilding. | Supplier and grade qualification; process-window changes. |
| Colour / L* / b* | Generally narrow, repeatable colour window. | Feedstock origin, ageing and contaminants can create greying/yellowing and batch variation. | Brand appearance, preform/bottle acceptance and blending limits. |
| Haze / clarity | Low and consistent for clear bottle grades. | Particle contamination and feedstock system can raise haze. A study reported 0.7% haze for virgin versus up to 6.6% for one 100% rPET series. [14] | Different clear-bottle and sheet qualification thresholds. |
| Black specks / particles | Lower risk from post-use foreign matter. | One study measured 14-40 times more insoluble particles in 100% rPET series than virgin material; collection route was decisive. [14] | More filtration, inspection and reject risk; visual defects. |
| Non-PET contaminants | Mainly production-control issue. | PVC, PP, PE, PS, labels, adhesives, metals, paper/cellulose and organics may enter through collection/sorting. [15] [16] | Requires feedstock specification, sorting and decontamination controls. |
| Moisture | Drying required before conversion. | Washing creates an additional moisture-control path; residual moisture can reduce IV during processing. | Drying discipline and incoming-material controls can differ. |
| AA / volatiles / NIAS | Controlled by resin and conversion conditions. | Additional NIAS (Non-Intentionally Added Substances) / contaminant pathways require process-specific risk assessment. Research found very low migration and low health concern in tested bottles, but distinct control needs. [15] | Separate food-contact evidence and customer approval. |
| Food-contact status | Conventional resin declaration and converter controls. | Authorised / validated recycling process, challenge tests, input provenance and critical operating parameters. [12] | Not every rPET grade is interchangeable with food-grade vPET. |
| Recycled-content proof | Not applicable. | Chain of custody, mass balance/physical traceability, batch documentation and auditable content evidence. [7] | Essential commercial attribute, tax evidence and customer claim support. |

## Balanced technical conclusion

The evidence does not establish that rPET is inherently unsuitable. It establishes that rPET quality is a distribution shaped by collection, sorting and recycling technology. Qualified food-grade rPET can meet demanding applications, but it requires additional controls and documentation and cannot be treated as commercially identical to vPET merely because both can become a bottle.

The commercial risk is greatest when the purchased specification does not adequately control contamination, viscosity, moisture, color, odor, traceability, and lot-to-lot consistency.

# COMMERCIAL IDENTITY, DEMAND AND SUBSTITUTION

## rPET is bought for a verified attribute that vPET cannot provide

UK Plastic Packaging Tax applies to finished plastic packaging with less than 30% recycled plastic and was introduced specifically to stimulate recycled-plastic demand and collection. The 2025/26 rate was £223.69/t and rose to £228.82/t from April 2026. HMRC also requires product-line records supporting recycled content. [7] [5]

| Market feature | vPET | rPET |
| --- | --- | --- |
| Reason for purchase | Polymer performance, availability and price. | Performance plus verified recycled content, customer targets, tax/circularity and brand commitments. |
| Substitutability | Can replace rPET technically in some blends/applications, subject to qualification. | Can replace some vPET in blends/applications, subject to quality and process limits. |
| Critical asymmetry | Cannot satisfy a recycled-content requirement. | Can carry the recycled-content attribute, but at a distinct quality, documentation and price profile. |
| Customer approval | Grade, food-contact and process qualification. | Those requirements plus recycled-content provenance, decontamination route and often tighter contaminant/colour review. |
| Supply response | Depends on petrochemical capacity and imports. | Depends on collection volumes, bale quality, sorting, approved capacity and yield. |
| End-market signals | Often linked to conventional PET and PTA/MEG economics. | Often linked to regulatory/brand demand and scarcity of food-grade recyclate. |
| Switching cost | Trials may be needed across preform, blow-moulding, filling and shelf-life systems. | Same trials plus recycled-content and chain-of-custody revalidation; switching to vPET may breach customer/claim requirements. |

## Policy is changing the demand curve

- Plastic Packaging Tax (PPT) has created an economic threshold at 30% recycled content; across all plastic packaging, HMRC reports that 51% of 2025/26 declared tonnage met the 30% threshold. This is not PET-specific, but it demonstrates the scale of policy-driven recycled-content demand.

- UK fiscal policy itself recognises recycled content as a distinct commercial attribute. Plastic packaging containing less than 30% recycled plastic is subject to Plastic Packaging Tax, currently £228.82 per tonne, with the rate having increased annually since introduction. From April 2027, the rules will further tighten by excluding pre-consumer waste from qualifying recycled content, while introducing a mass-balance framework for chemically recycled material. This reinforces the commercial importance of access to genuine recycled resin and demonstrates that vPET and rPET are not economically interchangeable merely because they share the same underlying polymer chemistry. [6]

- The planned Deposit Return Scheme from October 2027 covers PET drinks containers of 150 ml to 3 litres and is intended to generate higher-quality recycled material and support bottle-to-bottle recycling. However, collection alone does not immediately create sufficient food-grade rPET; material must still be sorted, decontaminated, reprocessed and qualified for food-contact use. A safeguard that raises the cost or limits availability of imported rPET could therefore work against DRS objectives by making recycled-content adoption more expensive during this transition. DRS will also add producer, retailer and system costs across the beverage value chain, creating additional pressure on consumer prices. Any safeguard should therefore be assessed carefully for its combined effect on circularity, rPET availability and downstream beverage costs [17]

## Commercial test for interchangeability

The correct question is not whether both resins can physically enter similar equipment. It is whether a buyer can switch without losing required recycled-content claims, food-contact approvals, customer acceptance, colour/contaminant performance, contracted sustainability targets and supply resilience. For many transactions, the answer is no.

The commercial risk is greatest when the purchased specification does not adequately control contamination, viscosity, moisture, color, odor, traceability, and lot-to-lot consistency.

# PRICING ARCHITECTURE AND PRICE TRANSMISSION

## Separate price series exist because the costs and markets differ

S&P Global assesses UK/Northwest European PET bottle bales, clear flakes, mixed-colour flakes and food-grade rPET pellets as distinct markets, and separately publishes spreads between rPET flakes, pellets and virgin PET. That market architecture is itself evidence that rPET is not merely a grade adjustment to vPET. [18] [19]

| Pricing layer | vPET mechanism | rPET mechanism |
| --- | --- | --- |
| Indicative cost stack | 0.84 x PTA + 0.34 x MEG + conversion/energy + logistics + margin. [3] | Recovered feedstock / mass yield + sort/wash + decontamination + extrusion/filtration + SSP + QA/certification + logistics. [20] [21] |
| Immediate supply driver | PTA/MEG and energy availability; operating rates of large continuous plants. | Bottle-bale/flake availability and quality; collection regime; contamination; food-grade capacity and yield. |
| Price relationship | A benchmark for conventional PET and often a reference point for rPET. | May trade at a premium or sometimes at a discount depending on policy demand, scarcity and virgin PET price; does not mechanically converge to vPET. |

## A concrete 2025 price snapshot

S&P Global reported a persistent and material European food-grade rPET premium to virgin PET during 2025. Premium to virgin started 2025 at $566/t, peaked at $836/t in early October, and ended the year at $723/t. [22]

# UK SUPPLY, DEMAND, CAPACITY AND MARKET EVOLUTION

## The UK has separate vPET and rPET capacity constraints

TRA estimated UK PET consumption at 370-420 kt in TS0060. APUK discloses 220 kt/y of Wilton PET capacity. At theoretical 100% utilisation, that nameplate is only 52-59% of the TRA demand range; actual output, grade mix, outages and exports reduce practical coverage.

An Enviroo analysis reported by RECOUP estimates UK food-grade rPET capacity at about 85 kt/y, rising to 120 kt by 2030, versus potential DRS PET bottle collections of 274 kt by 2030. These are industry estimates, but they point to a distinct recycling-capacity and feedstock-conversion challenge. [23]

| Evidence point | What is known | Analytical consequence |
| --- | --- | --- |
| Domestic production structure | APUK is the only known UK vPET producer and does not manufacture rPET; TS0060 identified multiple rPET producers but had limited participation/data. | A vPET producer injury record is not a substitute for an rPET industry injury record. |
| Imports | APUK reports aggregate PET imports rising from 163.8 kt in 2021 to 312.6 kt in 2025; 2025 is extrapolated from data through November. | Split by material type, IV, food-grade status and end use; test seasonality and stock-building. |
| Demand evolution | PPT began in 2022; DRS is planned for 2027. | rPET demand and feedstock are moving because of policy, not only price competition. |
| Supply resilience | SG0094 parties report UK vPET outages/force majeure and dependence on imported alternatives. | Imports may serve continuity and unmet specification needs, not simply displace domestic output. |
| Data range caution | Public estimates differ by definition: all PET, bottle-grade, food-grade pellet, flakes, production versus apparent consumption. | TRA should reconcile datasets rather than combine unlike figures. |

# CONSEQUENCES FOR IMPORT, INJURY AND REMEDY ANALYSIS

## A combined PET dataset can generate the wrong causal story

| Issue | Risk if vPET and rPET are combined | Required analysis |
| --- | --- | --- |
| Import surge | Policy-driven rPET growth may be counted as displacement of APUK vPET. | Import volumes and values by vPET/rPET, food-grade status, IV band, recycled-content share and end use. |
| Price effects | Premium food-grade rPET can distort average import prices and false undercutting comparisons. | Like-for-like price comparisons, including quality, recycled-content, Incoterms, duties and logistics. |
| Domestic industry | Injury to APUK may be treated as injury to UK rPET producers that use different assets/feedstocks. | Separate production, capacity, sales, employment, profitability and market share for each product group. |
| Causation | Outages, energy/feedstock volatility, demand changes and policy shifts may be attributed to imports. | Non-attribution analysis, with APUK outage/import periods and policy variables controlled. |
| Adjustment capacity | A vPET plant may be assumed able to replace rPET supply. | Evidence of actual ability to produce qualified food-grade rPET and secure compliant feedstock. |
| Economic interest | Benefits to one vPET plant may be compared with understated downstream/rPET costs. | Converter, filler, recycler, retailer and consumer impacts, including investment and supply continuity. |

## The legal/economic proposition

Direct competition in some applications is not enough to justify one import trend, one average price, one injury record and one remedy. The question is whether the evidence supports the same competitive conditions and injury pathway for both product groups. The present public record points in the opposite direction.

## Likely price effect of a broad safeguard

A broad restriction could reduce import competition in vPET, lift the reference benchmark used in rPET negotiations, and constrain imported food-grade rPET that the applicant does not produce. The combined effect may be higher prices across both markets, without resolving UK rPET feedstock and reprocessing constraints.

# RECOMMENDED TRA ANALYTICAL FRAMEWORK

## Minimum segmentation needed for a reliable determination

| Proposed category | Objective definition | Separate evidence required | Potential remedy treatment |
| --- | --- | --- | --- |
| vPET resin | Virgin-produced PET at the investigated IV threshold. | Imports, UK production, prices, sales, outages, utilisation and injury. | Independent duty/TRQ analysis. |
| Food-grade rPET pellet | Mechanically recycled PET pellet from documented post-consumer/industrial feedstock, with recognised food-contact process evidence and recycled-content traceability. | Bales/flakes, yield, capacity, food-grade approvals, prices, customer specifications and policy demand. | Exclude or establish separate PCN, injury test and remedy. |
| Non-food-grade rPET / flake | Recovered PET not qualified as food-grade pellet. | End use, upgrading requirement, import volume and direct competition. | Exclude where not directly competitive; otherwise separate PCN. |
| Specialty PET / copolyester | Grades materially differentiated by comonomer, processing or end use, including PETG where applicable. | Physical/functional properties, price and customer qualification. | Separate scope treatment. |

## Evidence TRA should request and publish in non-confidential form

- Monthly imports by product type, IV band and origin, with 2025 actuals.

- UK vPET and rPET capacity, actual output, grade mix and sales.

- Separate food-grade pellet, non-food pellet and flake volumes.

- Comparable delivered prices and terms, not aggregated customs values.

- Lost-sales allegations tied to specific product and qualification.

- APUK outage, force-majeure, imports and allocation periods.

- Domestic rPET feedstock, yield, authorised capacity and bottlenecks.

- PPT, pEPR, customer targets and DRS effects on demand/feedstock.

- Converter qualification, trial and switching costs.

- Employment, investment and finished-packaging import substitution.

## Customs-administerable evidence for rPET

A separate rPET category can be supported through supplier recycled-content declarations, batch COAs, chain-of-custody or mass-balance records, food-contact recycling-process authorisation, input-feedstock records and auditable production/yield records. HMRC already requires product-line evidence for recycled content under the Plastic Packaging Tax, and US PET scope practice demonstrates a content threshold can be expressed in written scope.

## OVERALL CONCLUSION

The evidence supports a finding of partial, asymmetric competition - not a single homogeneous market. vPET and rPET may meet overlapping packaging functions, yet their commercial identity is formed by different feedstocks, assets, cost curves, quality controls, regulatory obligations and supply constraints. Treating them as one product group would risk protecting vPET by taxing a distinct circular-economy input that APUK does not produce.

# References

| [1] | Alpek, "2025 Institutional Presentation," 2 2026. [Online]. Available: https://www.alpek.com/wp-content/uploads/2026/02/Alpek-4Q25-Institutional-Presentation.pdf. |
| --- | --- |
| [2] | Alpek, "2023 Institutional Presentation," 2 2024. [Online]. Available: https://www.alpek.com/wp-content/uploads/2024/02/Alpek-4Q23-Institutional-Presentation.pdf. |
| [3] | Alpek, "2024 Institutional Presentation," 2 2025. [Online]. Available: https://www.alpek.com/wp-content/uploads/2025/02/Alpek-4Q24-Institutional-Presentation.pdf. |
| [4] | TRA, "TRA, Final Determination, Transition Review TS0060: Countervailing duty on PET originating in India," 15 October 2025. [Online]. Available: https://afd-2xllir3jmyeq6-d9fkagedfdg4aabu.a02.azurefd.net/content/Final-determination_SD-0000020456-X9L1.pdf. |
| [5] | HMRC, "Plastic Packaging Tax statistics: background, rates and 30% recycled-content threshold," [Online]. Available: https://www.gov.uk/government/statistics/plastic-packaging-tax-ppt-statistics/plastic-packaging-tax-ppt-statistics-background-and-references. |
| [6] | HMRC, "Plastic Packaging Tax: mass balance approach and removal of pre-consumer plastic," 26 11 2025. [Online]. Available: https://www.gov.uk/government/publications/plastic-packaging-tax-mass-balance-approach-and-removal-of-pre-consumer-plastic. |
| [7] | HMRC, "Records and accounts required for Plastic Packaging Tax," [Online]. Available: https://www.gov.uk/guidance/record-keeping-and-accounts-for-plastic-packaging-tax. |
| [8] | NAPCOR, "Cradle-to-Resin Life Cycle Analysis of Polyethylene Terephthalate Resin - Final Revised Report," Franklin Associates, 2020. [Online]. Available: https://napcor.com/wp-content/uploads/2020/05/Final-Revised-Virgin-PET-Resin-LCA.pdf. |
| [9] | APR, "Life Cycle Impacts for Postconsumer Recycled Resins: PET, HDPE, and PP," Franklin Associates, 2018. [Online]. Available: https://napcor.com/wp-content/uploads/2020/05/LCA-2018-APR-Recycled-Resin-Report.pdf. |
| [10] | E. Bezeraj, "State-of-the-art of industrial PET mechanical recycling: technologies, impact of contamination and guidelines for decision-making," Royal Society of Chemistry, RSC Sustainability, 2025. [Online]. Available: https://pubs.rsc.org/su/article/3/5/1996/872830/State-of-the-art-of-industrial-PET-mechanical. |
| [11] | EFSA, "Safety assessment of the process Circular Plastics, based on the Starlinger iV+ technology, used to recycle post-consumer PET into food-contact materials," 2022. [Online]. Available: https://efsa.onlinelibrary.wiley.com/doi/10.2903/j.efsa.2022.7019. |
| [12] | EFSA, "Safety assessment of the process Plastrec, based on the Polymetrix technology, used to recycle post-consumer PET into food contact materials," EFSA Journal, 2023. [Online]. Available: https://efsa.onlinelibrary.wiley.com/doi/10.2903/j.efsa.2023.8149. |
| [13] | EFSA, "Safety assessment of the process rPET Aviv Shalam, based on the Starlinger iV+ technology, used to recycle post-consumer PET into food contact materials," 2022. [Online]. Available: https://efsa.onlinelibrary.wiley.com/doi/10.2903/j.efsa.2022.7670. |
| [14] | A. Chacon, "Effect of recycled content and rPET quality on the properties of PET bottles, Part I: Optical and mechanical properties," Packaging Technology and Science, 2020. [Online]. Available: https://doi.org/10.1002/pts.2490. |
| [15] | T. v. Velzen, "Effect of recycled content and rPET quality on the properties of PET bottles, Part II: Migration.," Packaging Technology and Science, 2020. [Online]. Available: https://doi.org/10.1002/pts.2528. |
| [16] | M. T. Brouwer, "Effect of recycled content and rPET quality on the properties of PET bottles, Part III: Modelling of repetitive recycling," Packaging Technology and Science, 2020. [Online]. Available: https://doi.org/10.1002/pts.2489. |
| [17] | DEFRA, "Deposit Return Scheme for drinks containers: policy statement, 7 May 2025," Department for Environment Food & Rural Affairs, 7 05 2025. [Online]. Available: https://www.gov.uk/government/publications/deposit-return-scheme-for-drinks-containers-policy-statement/deposit-return-scheme-for-drinks-containers-policy-statement. |
| [18] | "European Recycled PET Price Assessments: bales, flakes and food-grade pellets in NWE and the UK," S&P Global, [Online]. Available: https://www.spglobal.com/energy/en/pricing-benchmarks/assessments/chemicals/eu-recycled-pet-price-explained. |
| [19] | "Launches spreads for virgin and recycled plastics assessments," S&P Global Platts, 7 7 2021. [Online]. Available: https://www.spglobal.com/energy/en/pricing-benchmarks/our-methodology/subscriber-notes/070721-sampp-global-platts-launches-spreads-for-virgin-recycled-plastics-assessments. |
| [20] | E. Bezeraj, "State-of-the-art of industrial PET mechanical recycling: technologies, impact of contamination and guidelines for decision-making," RSC Sustainability, 2025. [Online]. Available: https://doi.org/10.1039/D4SU00571F. |
| [21] | K. Bataineh, "Life-Cycle Assessment of Recycling Postconsumer HDPE and PET," Advances in Civil Engineering, 2020. [Online]. Available: https://doi.org/10.1155/2020/8905431. |
| [22] | "European cost gaps, H1 2026 outlook, reporting selected 2025 rPET-vPET premiums," S&P Global, Recycled Polymers, [Online]. Available: https://www.spglobal.com/energy/en/news-research/special-reports/chemicals/chemical-trends-h1-2026/sustainability-in-peril/recycled-polymers. |
| [23] | "UK Needs £750M in New PET Recycling Capacity Ahead of Deposit Return Scheme," Resource Media, [Online]. Available: https://resourcemedia.eco/article/uk-needs-750m-in-new-pet-recycling-capacity-ahead-of-deposit-return-scheme. |